Colonic polyp detection using YOLO

Raluca Portase1, Eugen-Richard Ardelean1, *

1Department of Computer Science, Technical University of Cluj-Napoca, Cluj-Napoca, Romania

* Corresponding authors: ardeleaneugenrichard@gmail.com

ORCID Author IDs:

Raluca Portase: 0000-0002-8985-4728

Eugen-Richard Ardelean: 0000-0002-0098-4228

Abstract: Lately, deep learning has been applied more and more to medical image analysis. Specifically, object detection models may help in automatically identifying pathological structures and features. This study presents a comprehensive comparative analysis for object detection tasks in histological images of the latest models including the YOLO (You Only Look Once) architectures, from YOLOv8 to the recently introduced YOLOv12. These models were evaluated alongside alternative architectures including RT-DETR, YOLO-World, and YOLOE across three polyp detection datasets:

CVC-ClinicDB, CVC-ColonDB, ETIS-LaribPolypDB.

The experimental analysis employed standardized training protocols with consistent hyperparameters and data augmentation strategies, evaluating the performance through multiple metrics, inference time, and computational cost. The results obtained on the five datasets indicate that YOLOv11 consistently showed a strong performance across multiple datasets, however the newly introduced attention mechanisms of YOLOv12 show good performance, despite the model having slightly lower overall performance. Specialized variants like YOLOE demonstrated promising results for specific applications, while RT-DETR showed poor performance on smaller objects, which are typical in histological images. Statistical analyses indicate that YOLOv11 indeed has the best performance but that all models have a poor performance on objects of small sizes; moreover, the most common cases of failure are background false positives and missed detections. This comprehensive evaluation provides insights for the current state of object detection architectures for clinical histopathology applications and establishes benchmarks for future avenues of research in automated medical image analysis.

Keywords: YOLO, object detection, deep learning, medical image analysis, colorectal cancer, colonic polyp detection

# Introduction

## Colonoscopy

[1] Application of artificial intelligence in medicine is now attracting substantial attention. In the field of gastrointestinal endoscopy, computer-aided diagnosis (CAD) for colonoscopy is the most investigated area, although it is still in the preclinical phase.

Colorectal cancer (CRC) is a major cause of cancer-related death in both eastern and western countries.

Poorly conducted colonoscopy could impair CRC prevention. It has been established that the adenoma detection rate during colonoscopy is inversely associated with the incidence of post-colonoscopy CRC and with CRC-related mortality. Therefore, standardizing the quality of colonoscopy by reducing the number of missed adenomas/polyps is a desirable goal.

The other concern regarding the quality of the colonoscopy is the accuracy of the optical diagnosis of recognized colorectal polyps.

[2] Currently, artificial intelligence polyp detection technology is in its early stages of development. When compared to traditional statistics or expert systems, deep learning methods typically exhibit notable enhancements in performance and detection accuracy for most image target detection tasks

Gastrointestinal (GI) tract disorders are responsible for around 1 million deaths per year across Europe; and are associated with substantial morbidity and healthcare costs. The incidence and prevalence of many GI tract disorders are highest amongst the young and specially, the elderly, and as the world population ages, the disease burden will inevitably increase[3] [4].

Colorectal cancer (CRC) is the most common GI cancer in Europe, with a risk of CRC diagnosis in a lifetime of 1 in 20. The incidence in Europe is expected to increase from 3.6 million cases in 2015 to 4.3 million cases in 2035, due to increased life expectancy and adoption of western diet and lifestyle. Although the prognosis for early CRC diagnosis and 5-year survival can reach 70%, it stays extremely poor for late cases, being as low as 10%[3] [4].

[5]

Gastrointestinal tract disorders, including colorectal cancer (CRC), impose a significant health burden in Europe, with rising incidence rates among both young and elderly populations. Timely detection and removal of polyps, the precursors to CRC, are vital for prevention. Conventional colonoscopy, though effective, is prone to human errors.

Gastrointestinal (GI) tract disorders pose a significant public health concern in Europe, leading to approximately 1 million deaths annually. These disorders not only result in substantial mortality, but also impose a considerable burden of illness and healthcare expenses. It is noteworthy that the incidence and prevalence of many GI tract disorders are most prevalent among both the young and, particularly, the elderly population. Given the global aging trend, the burden of these diseases is expected to rise steadily in the future [3], [5]

Colorectal cancer (CRC) is a type of cancer affecting the large intestine and is among the most serious and prevalent forms of cancer. The 5-year survival rates are influenced by various factors and can vary significantly depending on the cancer stage and its location in either the colon or rectum. On average, 5-year survival rates for CRC are estimated to range from 48.6 to 59.4% [6]. Statistics project that by the year 2020, nearly 150,000 individuals will have been diagnosed with CRC, and more than 50,000 will succumb to the disease [7]. Colorectal cancer (CRC) is the second most common cause of cancer death in the United States. Every 3 years, the American Cancer Society provides an update of CRC occurrence based on incidence data (available through 2016) from population-based cancer registries and mortality data (through 2017) from the National Center for Health Statistics [7].

Notably, colorectal cancer has experienced the swiftest rise in incidence rates in recent times, with the number of new cases and deaths doubling over the past decade and continuing to increase at an average annual rate of 4–5%. Epidemiological studies reveal a concerning trend of CRC incidence among adults under the age of 50, with the numbers already significantly high and continuing to rise [8]. Research has shown that most CRC cases evolve gradually from colorectal polyps, particularly adenomatous polyps. Timely removal of these polyps through resection can effectively prevent the occurrence of CRC and reduce CRC-related mortality by up to 70% [1]. Addressing this preventive approach is crucial in combatting the increasing prevalence of colorectal cancer.

Colon cancer's survival rate depends on the stage in which it is detected, decreasing from rates higher than 95% in the first stages to rates lower than 35% in stages IV and V [9]; hence the importance of detecting it on its early stages by using screening techniques, such as colonoscopy [10].

Although colonoscopy is considered nowadays as the gold standard for colon screening there are still open challenges to overcome, such as the reduction of the miss-rate [11]. During the last decades there is a trend that consists of developing intelligent systems for medical applications. Intelligent systems are currently being used to assist in other medical interventions. For instance, there are systems that can interpret medical data automatically, such as KARDIO, which was developed to interpret electrocardiograms. It is possible to find many examples of intelligent systems built to assist in cancer detection. The interested reader can consult some works in the field of breast cancer detection [12] or prostate cancer detection [13].

[14]

Colonoscopy is the gold standard for colon cancer screening though some polyps are still missed, thus preventing early disease detection and treatment. Several computational systems have been proposed to assist polyp detection during colonoscopy but so far without consistent evaluation. The lack of publicly available annotated databases has made it difficult to compare methods and to assess if they achieve performance levels acceptable for clinical use. The Automatic Polyp Detection sub-challenge, conducted as part of the Endoscopic Vision Challenge (http://endovis.grand-challenge.org) at the international conference on Medical Image Computing and Computer Assisted Intervention (MICCAI) in 2015, was an effort to address this need. In this paper, we report the results of this comparative evaluation of polyp detection methods, as well as describe additional experiments to further explore differences between methods. We define performance metrics and provide evaluation databases that allow comparison of multiple methodologies. Results show that convolutional neural networks are the state of the art. Nevertheless, it is also demonstrated that combining different methodologies can lead to an improved overall performance.

## Neural networks in medical applications

[14]

This paper introduces the results and main conclusions of the MICCAI 2015 Sub-Challenge on Automatic Polyp Detection in Colonoscopy, conducted as part of the Endoscopic Vision Challenge (http://endovis.grand-challenge.org). More precisely, we present a validation study comparing the performance of different polyp detection methods covering different methodologies proposed by participating teams, providing an insight analysis of their detection yield.

We present in this paper a complete validation study comparing the performance of different polyp detection methods. Eight different teams took part in this challenge, ranging from methodologies based on hand-crafted methods to trending techniques such as CNNs. We propose the use of uniform performance metrics and common, publicly available, fully annotated databases to objectively assess their performance. The analysis of the results obtained by each method shows a superior performance by methods using machine learning as part of their methodologies, obtaining promising performance in both still frames and full-sequence sets. The global analysis of methods’ performance shows that some of them are close to be clinically applicable as they are able to detect polyps in all sequences with a small reaction time. We have also shown how there is a clear link between clinical and technical challenges and that mitigating them is key to improve methods’ performance. As it was expected, our preliminary study proves that image quality and careful mucosa inspection do have a positive impact in methods’ performance. A deep analysis of the results shows that, as different methods detect different polyps, there is room for improvement by combining some of the methods into a new solution. Going along this line, we have performed a first study on how to combine some of the methods in order to improve detection performance. Preliminary results show how the combination of the best methods can be used to exceed best individual scores, indicating the potential of creating clinically useful systems integrating capabilities from several individual methods.

The research community has identified the potential of deep learning algorithms for biomedical image analysis due to their efficient processing of complex data [15], [16]. The advancements in both hardware and deep learning architectures present an unprecedented potential to advance medical image analysis [15], with segmentation and object detection models showing particular promise [17], [18], [19] in general and for histopathological applications [20], [21], [22], [23]. Neural networks have been increasingly used in medical applications in recent years [24], [25] with newer and more performant approaches being created [30]. This is also supported by the substitution of image recognition techniques with deep learning approaches [26], [27], [28] including in pathological image analysis. Among these deep learning approaches, segmentation and object detection models have emerged as particularly useful for biomedical images analysis [15] with a three-fold increase between 2018 and 2022 as the third most popular task in medical images [25].

Several studies have demonstrated the effectiveness of object detection and segmentation models. In nucleus detection/segmentation with fluorescence microscopy, YOLO and U-Net [18] have been used with promising results. A combination of U-Net models has been developed for automatic tissue segmentation [29]. In OCT scans, U-Net and U-Net++ [30], models were used for the identification of biomarkers through the segmentation of retinal layers and fluids.

Object detection architectures can be divided into three categories: points-based detection models, two-stage detection models and single-stage detection models [31]. Perhaps the most known models throughout the large number of architectures are the "You Only Look Once" (YOLO) [31] family of models. The YOLO models have seen a great number of variants and versions through the years with new internal mechanisms for general improvements in performance or for specific tasks. Their notoriety can be attributed to their single-stage approach to detection which provides a balance between speed and accuracy, an important requirement for real-time analysis, as is the case in the biomedical field.

The YOLO architectures have been demonstrated to have applicability in a variety of medical tasks. YOLOv4 was applied for the nucleus detection in breast histopathological images [19]. YOLOv5 has shown a high performance for coronary OCT [32]. A modified YOLOv7 outperformed other traditional methods in the automated detection of blood vessels [33]. YOLOv8 and YOLOv9 have shown progress in feature detection for diabetic retinopathy from retinal fundus images with some limitations [34].

Several literature reviews [20], [35], [36], [37] have discussed the analysis of histopathological images including the use of machine learning algorithms. Machine learning has been identified as a promising avenue of research in histopathology due to its ability to uncover subtle patterns invisible to the human eye [38], [39] with the potential to trace cancer origins in histopathological images [38], [40]. Other studies [15] have identified the YOLO family of models as the most frequently used ranging across a wide variety of medical applications such as brain tumor detection, glaucoma detection, pulmonary embolism detection, skin lesion detection, kidney disease localization and many others [15]. The YOLO family of models was also identified as the most performant due to their robustness, high accuracy, and low computational cost [15].

YOLO has also been previously suggested and applied to histology [15]. YOLOv3 achieved a high performance in the detection of prostate cancer in histological images [41]. YOLOv4 was used in nucleus detection in breast histopathological images [14], as well as YOLOv5 [16]. A modification of YOLOv5 called Histology-based Detection using Yolo (HD-Yolo) [23] was designed for nucleus segmentation and tumor microenvironment characterization. YOLOv8 was applied for cochlea detection (using bounding boxes for localization) in macaque histology [41]. A hybrid pipeline was developed for fast cell detection using YOLOv11 and StarDist [42] for cell detection and segmentation that was shown to be highly performant on both both proprietary data and the public MoNuSeg benchmark dataset [43]. These deep learning approaches have shown promising results in pathology image analysis [20], [21], [22], [23] on various tasks including cell detection, nucleus detection [15], [18], [19], tissue segmentation, and cancer diagnosis [17], [38], demonstrating the potential to augment pathologist expertise, improve diagnostic accuracy and decrease the inherent time-cost.

To enhance the efficiency of polyp detection, several target detection algorithms based on the YOLO series have been developed. Guo et al. presented an automatic polyp detection algorithm utilizing the YOLO-V3 structure combined with active learning. This approach effectively reduces the false positive rate in polyp detection [44]. Cao et al. introduced a feature extraction and fusion module, integrating it with the YOLO-V3 network. By incorporating both high-level and low-level feature maps, this method can capture semantic information and outperforms other techniques in detecting small polyps [45]. Pacal et al. proposed a real-time automatic polyp detection method based on YOLO-V4. They integrated the cspnet network into the architecture and incorporated the mish activation function, Diou loss function, and transformer block. This approach demonstrates higher accuracy and superior performance compared to previous methods [46]. These advancements in target detection algorithms based on the YOLO series hold promise for more efficient and accurate polyp detection during colonoscopy. Lee and colleagues [47] introduced a real-time system for polyp detection utilizing YOLO-V4. The system employed a multiscale mesh for the identification of small polyps. Performance enhancements were achieved through the incorporation of advanced data augmentation techniques and the utilization of different activation functions. Wan and colleagues [48] proposed a model based on YOLO-V5 for real-time polyp detection, incorporating a self-attention mechanism. With this approach, the method strengthens beneficial features, weakens less relevant ones, resulting in an enhanced performance of polyp detection.

In a comprehensive experimental study, Pacal et al. [49] assessed novel datasets, SUN and PICCOLO, using the Scaled YOLO-V4 algorithm. The results of the experimental studies indicate that the SUN and PICCOLO datasets demonstrate exceptional success in polyp detection, with the Scaled YOLO-V4 algorithm standing out as one of the most suitable object detection algorithms for large-scale datasets. Durak and colleagues [50] conducted training on state-of-the-art object detection algorithms, including YOLO-V4, CenterNet, EfficientDet, and YOLO-V3, for automatic gastric polyp detection. In the experimental results, the YOLO-V4 algorithm demonstrated the highest performance compared to other methods, showcasing its effectiveness for deployment in CAD systems for automatic polyp detection. Qian et al. [51] proposed a method that combines GAN architectures with the YOLO-V4 object detection algorithm for robust polyp detection. Experimental evaluations were carried out on three publicly available datasets. The results indicated that the proposed method outperforms U-Net, synthesizes more realistic polyp images and significantly improves polyp detection performance. Gabriel provides a comprehensive implementation of YOLO-V4 at various precision levels (FP32, FP16, and INT8) for polyp detection. Notably, the study explores the previously untested INT8 quantization level. The research employs Darknet for YOLO-V4 training, integrates TensorRT for quantization and optimization (FP16 and INT8), and evaluates different data augmentation and regularization techniques on benchmark datasets of Etis-Larib and CVC-ClinicDB. The study achieves commendable performance metrics, including a median average precision (mAP) of 82.93% for Etis-Larib and 90.96% for CVC-ClinicDB. The analysis encompasses GPU specifications, inference speeds, and accuracy metrics for each precision level, revealing potential regularization effects with quantization. The findings contribute valuable insights to the application of quantization, particularly in the context of larger and more complex models like YOLO-V4, strengthening the argument for its role in network regularization [52]. In [53] by Ahmet Karaman and Ishak Pacal, the study introduces a groundbreaking integration of the ABC algorithm with YOLO-based object detection algorithms, focusing on optimizing activation functions and hyperparameters—an unprecedented exploration in the literature. This integration achieves more efficient optimization in a single operation, saving time and hardware costs. Key contributions include a 3% improvement in real-time polyp detection performance with the YOLO-V5 algorithm, marking the first demonstration of real-time capabilities on SUN and PICCOLO datasets. The study thoroughly examines the influence of activation functions and hyperparameters on real-time polyp detection accuracy. The proposed method is versatile, easily applicable to any dataset and YOLO-based algorithm, tailoring parameters to optimize performance [54].

[2]We constructed a diverse dataset from multiple publicly available sources and conducted extensive evaluations. YOLO-V8 m demonstrated impressive performance, achieving 95.6% precision, 91.7% recall, and 92.4% F1-score. It outperformed other state-of-the-art models in terms of mean average precision. YOLO-V8 s offered a balance between accuracy and computational efficiency. Our research provides valuable insights into enhancing polyp detection and contributes to the advancement of computer-aided diagnosis for colorectal cancer.

PD-YOLO [55] A colonoscopy is an important means of detecting colon polyps. However, in polyp imaging, due to the large differences and diverse types of polyps in size, shape, color, etc., traditional detection methods face the problem of high false positive rates, which creates problems for doctors during the diagnosis process. In order to improve the accuracy and efficiency of colon polyp detection, this question proposes a network model suitable for colon polyp detection (PD-YOLO). This method introduces the self-attention mechanism CBAM (Convolutional Block Attention Module) in the backbone layer based on YOLOv7, allowing the model to adaptively focus on key information and ignore the unimportant parts. To help the model do a better job of polyp localization and bounding box regression, add the SPD-Conv (Symmetric Positive Definite Convolution) module to the neck layer and use deconvolution instead of upsampling. The experimental results indicate that the PD-YOLO algorithm demonstrates strong robustness in colon polyp detection. Compared to the original YOLOv7, on the Kvasir-SEG dataset, PD-YOLO has shown an increase of 5.44 percentage points in AP@0.5, showcasing significant advantages over other mainstream methods.

[56]

Objectives: Timely and accurate detection of colorectal polyps plays a crucial role in diagnosing and preventing colorectal cancer, a major cause of mortality worldwide. This study introduces a new, lightweight, and efficient framework for polyp detection that combines the Local Outlier Factor (LOF) algorithm for filtering noisy data with the YOLO-v11n deep learning model.
Study design: An experimental study leveraging deep learning and outlier removal techniques across multiple public datasets.
Methods: The proposed approach was tested on five diverse and publicly available datasets: CVC-ColonDB, CVC-ClinicDB, Kvasir-SEG, ETIS, and EndoScene. Since these datasets originally lacked bounding box annotations, we converted their segmentation masks into suitable detection labels. To enhance the robustness and generalizability of our model, we apply 5-fold cross-validation and remove anomalous samples using the LOF method configured with 30 neighbors and a contamination ratio of 5%. Cleaned data are then fed into YOLO-v11n, a fast and resource-efficient object detection architecture optimized for real-time applications. We train the model using a combination of modern augmentation strategies to improve detection accuracy under diverse conditions.
Results: Our approach significantly improves polyp localization performance, achieving a precision of 95.83%, recall of 91.85%, F1-score of 93.48%, mAP@0.5 of 96.48%, and mAP@0.5:0.95 of 77.75%. Compared to previous YOLO-based methods, our model demonstrates enhanced accuracy and efficiency.
Conclusions: These results suggest that the proposed method is well-suited for real-time colonoscopy support in clinical settings. Overall, the study underscores how crucial data preprocessing and model efficiency are when designing effective AI systems for medical imaging.

[57]

Currently one stage models are prevalent due to their increased speed and compact nature and building on this the proposed model uses You Look Only Once (YOLO) for polyp detection. The novel module Wavelet-Enhanced Reverse Attention (WERA) along with a custom neck YOLO architecture makes it channel-efficient utilizing both the frequency and spatial domain coupled with feature fusion. The anchor-free head with mixed precision strategy results in a lightweight model. The model was tested on three datasets to test its reliability and performance in diverse environments. It was further tested on a combined dataset to test the generalisation capabilities. The proposed model achieves an increase of 1.98% to recall metrics and increase of 1.9% to the F2 score in the Polypgen dataset when compared with YOLO11 while still maintaining the precision.

[58]

Colorectal cancer (CRC) is the second leading cause of cancer-related deaths worldwide. Colonoscopy is the primary method to prevent CRC. However, traditional polyp detection methods face problems such as low image resolution and the possibility of missing polyps. In recent years, deep learning techniques have been extensively employed in the detection of colorectal polyps. However, these algorithms have not yet addressed the issue of detection in low-resolution images. In this study, we propose a novel YOLO-SRPD model by integrating SRGAN and YOLO to address the issue of low-resolution colonoscopy images. Firstly, the SRGAN with integrated ACmix is used to convert low-resolution images to high-resolution images. The generated high-resolution images are then used as the training set for polyp detection. Then, the C3_Res2Net is integrated into the YOLOv5 backbone to enhance multiscale feature extraction. Finally, CBAM modules are added before the prediction head to enhance attention to polyp information. The experimental results indicate that YOLO-SRPD achieves a mean average precision (mAP) of 94.2% and a precision of 95.2%. Compared to the original model (YOLOv5), the average accuracy increased by 1.8% and the recall rate increased by 5.6%. These experimental results confirm that YOLO-SRPD can address the low-resolution problem during colorectal polyp detection and exhibit exceptional robustness.

[59]

Colorectal (CRC) represents one of the leading global causes of cancer-related deaths since polyps function as original precursors to cancer development. The identification of colorectal polyps at an early stage directly contributes to lowering CRC incidence while giving patients improved survival outcomes. The proposed research utilizes YOLOv8m through YOLOv11m state-of-the-art YOLO architectures as an advanced framework to enhance real-time detection and classification of polyps. The research used four different colorectal database sets including Kvasir-SEG and CVC-300, CVC-ClinicDB, CVC-ColonDB for training and validating the models across a wide spectrum of polyp image variations. Following 300 training epochs YOLOv8m achieved the best performance with precision at 92.4% and recall at 85.4% while reaching an F1-Score of 88.7% and mAP@0.5 at 91.1%. The testing of YOLOv8m’s generalization skills included an external validation on ETIS-LaribPolypDB where it demonstrated reliable detection performance and high accuracy levels. YOLOv8m demonstrates a strong capability for real-time polyp detection in endoscopic examinations through these performance results which showcase its potential to enhance diagnostic support.

[60]

Colon cancer is a prevalent malignancy, substantially it prevented most effectively from killing patients through early endoscopic detection. With the rapid development of artificial intelligence technology, the early diagnosis rate of colonic polyps achieves greater clinical efficacy for colon cancer by applying target detection algorithms to colonoscopy images. This paper presents two outcomes achieved through the application of the improved YOLOv5s algorithm with annotated microscopy images of clinical cases and publicly available polyp image data: (1) enhancement of the C3(Cross Stage Partial Networks) module with multiple layers to C3SE(Cross Stage Partial Networks with Squeeze-and-Excitation) via the attention mechanism SE (squeeze-and-excitation) and (2) fusion of higher-level features utilizing BiFPN (the weighted bi-directional feature pyramid network). Experimental comparisons are performed based on a new image dataset of colonic polyps among more than 6 target detection algorithms to validate the better detection capability. The tests indicate that the YOLOv5s + BiFPN and YOLOv5s-1st-2nd-C3SE models exhibit enhancements of detection capability compared to the YOLOv5 algorithm according to the main indicators of the mAP, accuracy, and recall. The YOLOv5s + SEBiFPN model demonstrate a substantial improvement over the YOLOv5s algorithm, and establishing a benchmark technology for advancing computer-assisted diagnostic systems is feasible.

A selection of highly performant object detection models from the latest YOLO versions from YOLOv8-v12 against alternative architectures such as RT-DETR, YOLO-World, YOLOE were evaluated. These models were evaluated on five diverse histological datasets from the perspective of five performance metrics (mAP@50, mAP@50-95, F1 score, precision and recall) and computational efficiency (inference time, FLOPs). A standardized parametrization was used in the analysis to guarantee a fair comparison including hyperparameter optimization and data augmentation.

Our contributions include: (1) benchmarking of the latest YOLO versions from YOLOv8-v12 against alternative architectures such as RT-DETR, YOLO-World, YOLOE across five diverse datasets for object detection of cancerous regions in histopathological images, (2) an analysis of performance trade-offs between accuracy, speed, and computational efficiency in the clinical context of the data, (3) statistical analysis to determine detection capability, distribution of failures and statistical ranking, (4) a graph-based method for test-time inference refinement and its performance evaluation and (5) insights for the deployment of automated histology analysis systems in clinical settings.

# Materials and Methods

## Datasets

Table 1 – Short description of datasets.

| Dataset | Organs | Annotation type | Image count | Classes | Origin |
| --- | --- | --- | --- | --- | --- |
| CVC-ClinicDB | Colon (colonoscopy still frames) | PNG mask | 612 | 1 (polyp) | Computer Vision Center and Universitat Autònoma de Barcelona (Spain) |
| CVC-ColonDB | Colon | PNG mask | 380 | 1 (polyp) | Computer Vision Center and Universitat Autònoma de Barcelona (Spain) |
| ETIS-LaribPolypDB | Colon | TIF mask | 156 | 1 (polyp) | ETIS, ENSEA, University of Cergy-Pontoise, CNRS, Cergy (France) |

### CVC-ClinicDB

CVC-ClinicDB [62]contains 612 Standard Definition (SD) frames and comprises 31 different polyps from 31 sequences.

This method [14] is based on a model of appearance considering polyps as protruding surfaces, being their boundaries defined from intensity valleys detection. Their proposal includes a pre-preprocessing stage to mitigate the impact of other valley-rich structures (blood vessels, specular highlights). To build final energy maps highlighting polyp presence, four different constraints (continuity, completeness, concavity, and robustness against spurious structures) are imposed to candidate boundaries to differentiate polyps from other structures.

Both datasets gave each image a border, specular, lumen, and segmentation mask. The border mask marks the black border around each image, the specular mask indicates the reflections that come from the endoscope light, and the lumen mask labels the intestinal lumen, which is the space within an intestine. The segmentation mask contains polyp markings that tag visible polyps within a picture. Because we needed the bounding box from a polyp, we only used the segmentation masks and extracted a bounding box by calculating a box that fits around a single blob. The dataset CVC-ColonDB contains 300 selected images from 13 polyp video sequences with a resolution of 574 × 500 and CVC-Clinic-DB holds 612 images from 31 polyp video sequences with a size of 348 × 288 pixels. This dataset contains no healthy mucosa images. The data are available on request in the CVC-Colon repository (http://www.cvc.uab.es/CVC-Colon/index.php/databases/

### CVC-ColonDB

CVC-ColonDB [9]

### ETIS-LaribPolypDB

ETIS-LaribPolypDB [63]

ETIS-LaribPolypDB is from the MICCAI 2015 Endoscopic Vision Challenge and was used as the testing dataset in the challenge. Here, we include this dataset in our training dataset. It has 196 polyp images with the corresponding mask for boxes. For our training, we extracted the bounding boxes from the segmentation masks. The size of the images is 348 × 288 pixels. This dataset contains no healthy mucosa images. This dataset contains no healthy mucosa images. The data are available on request in the CVC-Colon repository (http://www.cvc.uab.es/CVC-Colon/index.php/databases

ETIS-LARIB database contains 196 High Definition (HD) frames and comprises 44 different polyps from 34 sequences

This method [26] is inspired by the psycho-visual methodology used by clinicians when performing an endoscopic examination. First, a detection of the Regions of Interests (ROI) that may contain a polyp is performed using shape and size image features. This first pre-selection allows a first and fast scanning of the image. Due to being circular/elliptical shapes associated to polyps, a Hough transform was used for this first filtering stage. Once ROIs are detected, a second analysis, based on texture is achieved in order to remove those ROIs with no actual polyp content. To achieve this, an ad-hoc classifier based on a boosting-based learning process using texture features computed from co-ocurrence matrices (standard Haralick features) is proposed

## Object detection models

The "You Only Look Once" (YOLO) [31] algorithm revolutionized real-time object detection through the unification of region proposal and classification. YOLO divides an image through a grid attempting to predict a class and a bounding box for each cell of the grid. This approach significantly reduced computation time and enabled end-to-end learning. YOLOv1 managed to achieve real-time detection although it incorporated a CNN backbone; however, small objects were hard to identify and localize accurately due to the fact that each grid cell was only able to predict two bounding boxes.

Since the introduction of YOLO in 2015, the YOLO family of algorithms [64] has expanded both in depth and width with several incremental improvements and various modified versions for specific applications. Each new version introduced advancements with the goal of improving computational efficiency and detection accuracy. YOLOv2 [64], [65], [66] brought several enhancements to YOLOv1 including higher input resolution, batch normalization, and the adoption of anchor boxes from Faster R-CNN to improve training and performance. YOLOv3 [64], [65], [67] achieved higher accuracy in small-object detection through multi-scale predictions using Feature Pyramid Networks (FPN) and a deeper architecture, specifically Darknet-53. YOLOv4 [64], [65], [68] managed to set new standards for detection accuracy and inference speed through several new introductions, such as: CSPDarknet-53, PANet, Mish activation, and data augmentation methods such as Mosaic and CutMix. YOLOv5 by Ultralytics [64], [65], [69] was designed with a PyTorch-based framework with an emphasis on usability and performance and made several changes such as: enhanced backbone, neck, and head designs, and an auto-anchor learning mechanism that optimized anchor sizes during training. YOLOv6 [64], [65], [70] added the EfficientRep backbone and RepOptimizer to improve efficiency. YOLOv7 [64], [65], [71] improved efficiency by introducing a trainable bag-of-freebies and the Extended Efficient Layer Aggregation Network (E-ELAN).

YOLOv8 [64], [65], [69], also from Ultralytics [69], integrated the C2f module into the architecture and transitioned to anchor-free detection for a more simplified and specialized detection. YOLOv9 [64], [65], [72] incorporated the Generalized Efficient Layer Aggregation Network (GELAN) and Programmable Gradient Information (PGI) to mitigate information degradation in deep networks. YOLOv10 [64], [65], [73] removed non-maximum suppression (NMS) and introduced a dual assignment strategy and lightweight classification heads achieving real-time efficiency. YOLOv11 [64], [65], [69] integrated the C3k2 block, SPPF, and C2PSA into the architecture to improve feature extraction and spatial attention while balancing precision and efficiency.

YOLOv12 [64], [65], [69], [74] shifted toward an attention-centric architecture achieving state-of-the-art accuracy and efficiency while maintaining real-time performance [65], [74]. The innovations that allowed YOLOv12 to achieve this (confirmed through ablation studies [74]) are: the Area Attention module, which reduces computational complexity while maintaining a large receptive field, and Residual Efficient Layer Aggregation Networks (R-ELAN) that enhance feature aggregation and optimize training for larger models. Other architectural updates include FlashAttention for memory optimization, MLP ratio adjustments, attention-convolution integration for efficiency, removal of positional encoding, and the Position Perceiver using large separable convolutions for positional awareness. Extensive analyses confirmed YOLOv12’s performance [74] showing increased accuracy, speed, and reduced computational cost compared to YOLOv6, YOLOv8, YOLOv10, YOLOv11, and RT-DETR variants. Despite its recent introduction in February 2025 [74], YOLOv12 has been successfully applied to early detection of sexually transmitted diseases [75], UAV tracking of [76], fruit detection [77], [78], and marine litter detection [79], [80] outperforming earlier YOLO versions.

YOLO-World [81] was derived from YOLOv8 and it added open-vocabulary detection using vision-language modeling demonstrating increased performance in real-world applications and in zero-shot scenarios for a wide range of objects. YOLO-World allows for the interaction between visual and linguistic information through the Re-parameterizable Vision-Language Path Aggregation Network (RepVL-PAN) and the alignment of this information through region-text contrastive loss, enhancing zero-shot detection for diverse applications.

YOLOE [82] was derived from YOLOv8 and YOLOv11 and it added prompt-based mechanisms for both object detection and instance segmentation which allows for the integration of contextual information; thus, enhancing preprocessing with context-relevant information [83]. It supports text prompts via the Re-parameterizable Region-Text Alignment (RepRTA) strategy, visual prompts via the Semantic Activated Visual Prompt Encoder (SAVPE), and no prompts via the Lazy Region-Prompt Contrast (LRPC) strategy.

Real-Time DEtection TRansformer (RT-DETR) [78], [84], [85] is an alternative approach to object detection based on an end-to-end transformer removing the need for non-maximum suppression. RT-DETR replaces the standard Transformer encoder with a hybrid encoder to process intra-scale interactions and cross-scale feature fusion separately, increasing speed and detection accuracy via a query selection mechanism. RT-DETRv2 [85] achieves increased multi-scale detection through several modifications: dynamic data augmentation, scale-adaptive hyperparameters, and Selective Multi-Scale Sampling.

The YOLO family of models are released in multiple model scales offering trade-offs between accuracy, speed, computational cost, and parameter count. They are represented by suffixes: nano (n), small (s), medium (m), large (l), extra large (x). Larger models provide higher accuracy at increased computational cost, allowing users to select versions based on specific application needs, from maximum speed (n) to highest precision (x).

## Performance evaluation

Precision represents the percentage of positive predictions that are correct and is calculated as the ratio of true positives (TP) to the sum of true positives and false positives (FP). Precision measures the ability to avoid false alarms and is computed as:

Precision= TPTP+FP

Where TP represents the correct detections compared to the ground truth and FP represents the incorrect detections where no object actually exists.

Recall represents the percentage of actual positives correctly identified and is calculated as the ratio of true positives to the sum of true positives and false negatives (FN). Recall measures the ability to avoid missed detections and is computed as:

Recall= TPTP+FN

Where TP represents the correct detections compared to the ground truth and FN represents the cases where an object is missed compared to the ground truth.

The F1 score is considered a balanced measure of performance as it computed as the harmonic mean of precision and recall. In contrast to precision and recall, the F1 score incorporates both for false positives and false negatives. This makes it particularly useful for the evaluations of performance in applications where correctly identifying positive cases is as important as minimizing false alarms. It is computed as:

F1= 2*Precision*RecallPrecision+Recall

Intersection over Union (IoU) represents the overlap in area between predicted and true bounding boxes. IoU assesses how accurately the model is able to capture object boundaries and is computed as:

IoU= Intersection areaUnion area

Mean Average Precision at an IoU threshold of 0.5 (mAP@50) represents the area under the precision-recall curve, also known as the Average Precision (AP), computed for each class where detections achieve an overlap with with true bounding boxes at least 0.5 IoU. It is computed as:

mAP@50= 1|C|c∈CAPc(IoU≥0.5)

Where c represents a single class of from the set of classes C.

Mean Average Precision across IoU thresholds from 0.5 to 0.95 (mAP@50–95) represents AP scores averaged across multiple IoU thresholds ranging from 0.5 to 0.95 in increments of 0.05. This metric reflects localization performance under varying strictness levels, penalizing detections that only loosely match true objects. It is computed as:

mAP@50-95= 1|C|c∈C(1|T|t∈TAPc(IoU≥t))

Where c represents a single class of from the set of classes C, and t represents a single threshold from the set of thresholds T ={0.5, 0.55, ..., 0.95}.

Although commonly practiced, evaluating performance using a single performance metric can be risky endeavor [86], [87]. Especially if the metric fails to capture all relevant aspects, potentially leading to misleading and biased results. Therefore, we employed multiple complementary metrics to provide a more thorough evaluation across different aspects of performance.

# Results

## Performance on individual datasets

All models were trained using identical experimental conditions to ensure fair comparison. The training procedure used the same random subset of 70% of the original dataset for training, while validation and testing subsets contained 15% and 15%. All results presented are on the previously-unseen non-augmented test sets.

Models were trained for 50 epochs across the entire training set with batches of 16 images rescaled to 640×640 pixels. The AdamW optimizer was used with a starting learning rate of 1e-4, which was reduced to 1e-6 during the training process. A small penalty to large weights was added through a weight decay of 5e-4. A training learning rate warmup of 10 epochs was used to stabilize training. Data augmentation was applied to reduce the possibility of overfitting. Training images in HSV color space were randomly adjusted with hue, saturation, and value shifts of 0.015, 0.7, and 0.4, respectively. Additional augmentations included random rotations of up to ±10°, translations of up to 10% along either axis, and random zoom variations of up to 20%. Mixup augmentation was applied during training, blending two images into a single composite to enhance generalization by introducing additional noise and variability. Mosaic augmentation was also employed to combine multiple images, simulating visually complex scenarios. For model stability, mosaic augmentation was disabled during the final 10 training epochs.

The comparative evaluation of object detection models across the datasets showed considerable variations in performance depending on the dataset characteristics and models. Highlighting importance of considering multiple evaluation metrics and dataset- and domain-specific challenges when selecting object detection models for specific applications. The comparative evaluation was performed on the previously unseen non-augmented test partitions of the datasets.

### CVC-ClinicDB

Table – Results of object detection models on the dataset.

| Model/Metric | mAP@50 | mAP@50-95 | Mean F1 | Mean Precision | Mean Recall | Inference Time [ms] | Computational Cost [GFLOPs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RT-DETR |  |  |  |  |  |  | 103.4 |
| YOLOv8 |  |  |  |  |  |  | 28.4 |
| YOLOv9 |  |  |  |  |  |  | 26.7 |
| YOLOv10 |  |  |  |  |  |  | 24.5 |
| YOLOv11 |  |  |  |  |  |  | 21.3 |
| YOLOv12 |  |  |  |  |  |  | 21.2 |
| YOLOE |  |  |  |  |  |  | 35.3 |
| YOLOv8-Worldv2 |  |  |  |  |  |  | 32.6 |
| YOLOv26 |  |  |  |  |  |  |  |

### CVC-ColonDB

Table – Results of object detection models on the dataset.

| Model/Metric | mAP@50 | mAP@50-95 | Mean F1 | Mean Precision | Mean Recall | Inference Time [ms] | Computational Cost [GFLOPs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RT-DETR |  |  |  |  |  |  | 103.4 |
| YOLOv8 |  |  |  |  |  |  | 28.4 |
| YOLOv9 |  |  |  |  |  |  | 26.7 |
| YOLOv10 |  |  |  |  |  |  | 24.5 |
| YOLOv11 |  |  |  |  |  |  | 21.3 |
| YOLOv12 |  |  |  |  |  |  | 21.2 |
| YOLOE |  |  |  |  |  |  | 35.3 |
| YOLOv8-Worldv2 |  |  |  |  |  |  | 32.6 |
| YOLOv26 |  |  |  |  |  |  |  |

### ETIS-LaribPolypDB

Table – Results of object detection models on the dataset.

| Model/Metric | mAP@50 | mAP@50-95 | Mean F1 | Mean Precision | Mean Recall | Inference Time [ms] | Computational Cost [GFLOPs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RT-DETR |  |  |  |  |  |  | 103.4 |
| YOLOv8 |  |  |  |  |  |  | 28.4 |
| YOLOv9 |  |  |  |  |  |  | 26.7 |
| YOLOv10 |  |  |  |  |  |  | 24.5 |
| YOLOv11 |  |  |  |  |  |  | 21.3 |
| YOLOv12 |  |  |  |  |  |  | 21.2 |
| YOLOE |  |  |  |  |  |  | 35.3 |
| YOLOv8-Worldv2 |  |  |  |  |  |  | 32.6 |
| YOLOv26 |  |  |  |  |  |  |  |

## Statistics across datasets

# Conclusions

[1]	S. Kudo et al., “Artificial intelligence and colonoscopy: Current status and future perspectives,” Digestive Endoscopy, vol. 31, no. 4, pp. 363–371, 2019, doi: 10.1111/den.13340.

[2]	M. Lalinia and A. Sahafi, “Colorectal polyp detection in colonoscopy images using YOLO-V8 network,” SIViP, vol. 18, no. 3, pp. 2047–2058, Apr. 2024, doi: 10.1007/s11760-023-02835-1.

[3]	A. Sahafi et al., “Edge artificial intelligence wireless video capsule endoscopy,” Sci Rep, vol. 12, no. 1, p. 13723, Aug. 2022, doi: 10.1038/s41598-022-17502-7.

[4]	L. Bujanda, A. Cosme, I. Gil, and J. I. Arenas-Mirave, “Malignant colorectal polyps,” World J Gastroenterol, vol. 16, no. 25, pp. 3103–3111, Jul. 2010, doi: 10.3748/wjg.v16.i25.3103.

[5]	M. Lalinia and A. Sahafi, “Colorectal polyp detection in colonoscopy images using YOLO-V8 network,” SIViP, vol. 18, no. 3, pp. 2047–2058, Apr. 2024, doi: 10.1007/s11760-023-02835-1.

[6]	J. Lewis, Y.-J. Cha, and J. Kim, “Dual encoder–decoder-based deep polyp segmentation network for colonoscopy images,” Sci Rep, vol. 13, no. 1, p. 1183, Jan. 2023, doi: 10.1038/s41598-023-28530-2.

[7]	R. L. Siegel et al., “Colorectal cancer statistics, 2020,” CA Cancer J Clin, vol. 70, no. 3, pp. 145–164, May 2020, doi: 10.3322/caac.21601.

[8]	E. M. Stoffel and C. C. Murphy, “Epidemiology and Mechanisms of the Increasing Incidence of Colon and Rectal Cancers in Young Adults,” Gastroenterology, vol. 158, no. 2, pp. 341–353, Jan. 2020, doi: 10.1053/j.gastro.2019.07.055.

[9]	J. Bernal, J. Sánchez, and F. Vilariño, “Towards automatic polyp detection with a polyp appearance model,” Pattern Recognition, vol. 45, no. 9, pp. 3166–3182, Sep. 2012, doi: 10.1016/j.patcog.2012.03.002.

[10]	J. P. Hassinger, S. D. Holubar, R. Pendlimari, E. J. Dozois, D. W. Larson, and R. R. Cima, “Effectiveness of a multimedia-based educational intervention for improving colon cancer literacy in screening colonoscopy patients,” Dis Colon Rectum, vol. 53, no. 9, pp. 1301–1307, Sep. 2010, doi: 10.1007/DCR.0b013e3181e291c0.

[11]	L. von Karsa, J. Patnick, and N. Segnan, “European guidelines for quality assurance in colorectal cancer screening and diagnosis. First Edition--Executive summary,” Endoscopy, vol. 44 Suppl 3, pp. SE1-8, Sep. 2012, doi: 10.1055/s-0032-1309822.

[12]	J. Wei et al., “Computer-aided detection of breast masses: Four-view strategy for screening mammography,” Med Phys, vol. 38, no. 4, pp. 1867–1876, Apr. 2011, doi: 10.1118/1.3560462.

[13]	S. Viswanath et al., “Empirical evaluation of bias field correction algorithms for computer-aided detection of prostate cancer on T2w MRI,” in Medical Imaging 2011: Computer-Aided Diagnosis, SPIE, Mar. 2011, pp. 254–265. doi: 10.1117/12.878813.

[14]	J. Bernal et al., “Comparative Validation of Polyp Detection Methods in Video Colonoscopy: Results From the MICCAI 2015 Endoscopic Vision Challenge,” IEEE Transactions on Medical Imaging, vol. 36, no. 6, pp. 1231–1249, Jun. 2017, doi: 10.1109/TMI.2017.2664042.

[15]	F. Wahid, Y. Ma, D. Khan, M. Aamir, and S. U. K. Bukhari, “Biomedical Image Segmentation: A Systematic Literature Review of Deep Learning Based Object Detection Methods,” arXiv.org. Accessed: Jun. 03, 2025. [Online]. Available: https://arxiv.org/abs/2408.03393v2

[16]	P. M. Cheng et al., “Deep Learning: An Update for Radiologists,” Radiographics, vol. 41, no. 5, pp. 1427–1445, 2021, doi: 10.1148/rg.2021200210.

[17]	A. Bal, M. Das, and S. M. Satapathy, “YOLO as a Region Proposal Network for Diagnosing Breast Cancer,” in 2021 Grace Hopper Celebration India (GHCI), Feb. 2021, pp. 1–6. doi: 10.1109/GHCI50508.2021.9513988.

[18]	H. Narotamo, J. M. Sanches, and M. Silveira, “Segmentation of Cell Nuclei in Fluorescence Microscopy Images Using Deep Learning,” in Pattern Recognition and Image Analysis, A. Morales, J. Fierrez, J. S. Sánchez, and B. Ribeiro, Eds., Cham: Springer International Publishing, 2019, pp. 53–64. doi: 10.1007/978-3-030-31332-6_5.

[19]	L. S. Nair, R. P. R, G. Sugathan, K. V. Gireesh, and A. S. Nair, “Mitotic Nuclei Detection in Breast Histopathology Images using YOLOv4,” in 2021 12th International Conference on Computing Communication and Networking Technologies (ICCCNT), Jul. 2021, pp. 1–5. doi: 10.1109/ICCCNT51525.2021.9579969.

[20]	G. Litjens et al., “A survey on deep learning in medical image analysis,” Medical Image Analysis, vol. 42, pp. 60–88, Dec. 2017, doi: 10.1016/j.media.2017.07.005.

[21]	S. Wang, D. M. Yang, R. Rong, X. Zhan, and G. Xiao, “Pathology Image Analysis Using Segmentation Deep Learning Algorithms,” The American Journal of Pathology, vol. 189, no. 9, pp. 1686–1698, Sep. 2019, doi: 10.1016/j.ajpath.2019.05.007.

[22]	A. Janowczyk and A. Madabhushi, “Deep learning for digital pathology image analysis: A comprehensive tutorial with selected use cases,” Journal of Pathology Informatics, vol. 7, no. 1, p. 29, Jan. 2016, doi: 10.4103/2153-3539.186902.

[23]	R. Rong et al., “A Deep Learning Approach for Histology-Based Nucleus Segmentation and Tumor Microenvironment Characterization,” Mod Pathol, vol. 36, no. 8, p. 100196, Aug. 2023, doi: 10.1016/j.modpat.2023.100196.

[24]	F. M. Howard et al., “Generative adversarial networks accurately reconstruct pan-cancer histology from pathologic, genomic, and radiographic latent features,” Science Advances, vol. 10, no. 46, p. eadq0856, Nov. 2024, doi: 10.1126/sciadv.adq0856.

[25]	Y. Ma, S. Jamdade, L. Konduri, and H. Sailem, “AI in Histopathology Explorer for comprehensive analysis of the evolving AI landscape in histopathology,” npj Digit. Med., vol. 8, no. 1, p. 156, Mar. 2025, doi: 10.1038/s41746-025-01524-2.

[26]	F. Sheikhzadeh, R. K. Ward, D. van Niekerk, and M. Guillaud, “Automatic labeling of molecular biomarkers of immunohistochemistry images using fully convolutional networks,” PLOS ONE, vol. 13, no. 1, p. e0190783, Jan. 2018, doi: 10.1371/journal.pone.0190783.

[27]	J. Xu, X. Luo, G. Wang, H. Gilmore, and A. Madabhushi, “A Deep Convolutional Neural Network for segmenting and classifying epithelial and stromal regions in histopathological images,” Neurocomputing, vol. 191, pp. 214–223, May 2016, doi: 10.1016/j.neucom.2016.01.034.

[28]	L. Hou, D. Samaras, T. M. Kurc, Y. Gao, J. E. Davis, and J. H. Saltz, “Patch-based Convolutional Neural Network for Whole Slide Tissue Image Classification,” Proc IEEE Comput Soc Conf Comput Vis Pattern Recognit, vol. 2016, pp. 2424–2433, 2016, doi: 10.1109/CVPR.2016.266.

[29]	J. van der Laak, G. Litjens, and F. Ciompi, “Deep learning in histopathology: the path to the clinic,” Nat Med, vol. 27, no. 5, pp. 775–784, May 2021, doi: 10.1038/s41591-021-01343-4.

[30]	R. Brehar, A. Groza, I. Damian, G. Muntean, and S. D. Nicoară, “Age-Related Macular Degeneration Biomarker Segmentation from OCT images,” in 2023 24th International Conference on Control Systems and Computer Science (CSCS), May 2023, pp. 444–451. doi: 10.1109/CSCS59211.2023.00076.

[31]	J. Redmon, S. Divvala, R. Girshick, and A. Farhadi, “You Only Look Once: Unified, Real-Time Object Detection,” May 09, 2016, arXiv: arXiv:1506.02640. doi: 10.48550/arXiv.1506.02640.

[32]	H. Liu et al., “Toward reliable calcification detection: calibration of uncertainty in object detection from coronary optical coherence tomography images,” J Biomed Opt, vol. 28, no. 3, p. 036008, Mar. 2023, doi: 10.1117/1.JBO.28.3.036008.

[33]	C. Li et al., “PI-YOLO: dynamic sparse attention and lightweight convolutional based YOLO for vessel detection in pathological images,” Front. Oncol., vol. 14, Jul. 2024, doi: 10.3389/fonc.2024.1347123.

[34]	N. Rizzieri, L. Dall’Asta, and M. Ozoliņš, “Diabetic Retinopathy Features Segmentation without Coding Experience with Computer Vision Models YOLOv8 and YOLOv9,” Vision, vol. 8, no. 3, Art. no. 3, Sep. 2024, doi: 10.3390/vision8030048.

[35]	M. N. Gurcan, L. E. Boucheron, A. Can, A. Madabhushi, N. M. Rajpoot, and B. Yener, “Histopathological Image Analysis: A Review,” IEEE Reviews in Biomedical Engineering, vol. 2, pp. 147–171, 2009, doi: 10.1109/RBME.2009.2034865.

[36]	D. Shen, G. Wu, and H.-I. Suk, “Deep Learning in Medical Image Analysis,” Annual Review of Biomedical Engineering, vol. 19, no. Volume 19, 2017, pp. 221–248, Jun. 2017, doi: 10.1146/annurev-bioeng-071516-044442.

[37]	F. Xing and L. Yang, “Robust Nucleus/Cell Detection and Segmentation in Digital Pathology and Microscopy Images: A Comprehensive Review,” IEEE Reviews in Biomedical Engineering, vol. 9, pp. 234–263, 2016, doi: 10.1109/RBME.2016.2515127.

[38]	G. Nir et al., “Automatic grading of prostate cancer in digitized histopathology images: Learning from multiple experts,” Medical Image Analysis, vol. 50, pp. 167–180, Dec. 2018, doi: 10.1016/j.media.2018.09.005.

[39]	D. Komura and S. Ishikawa, “Machine Learning Methods for Histopathological Image Analysis,” Computational and Structural Biotechnology Journal, vol. 16, pp. 34–42, Jan. 2018, doi: 10.1016/j.csbj.2018.01.001.

[40]	M. Y. Lu et al., “AI-based pathology predicts origins for cancers of unknown primary,” Nature, vol. 594, no. 7861, pp. 106–110, Jun. 2021, doi: 10.1038/s41586-021-03512-4.

[41]	M. E. Salman, G. Çakirsoy Çakar, J. Azimjonov, M. Kösem, and İ. H. Cedi̇moğlu, “Automated prostate cancer grading and diagnosis system using deep learning-based Yolo object detection algorithm,” Expert Systems with Applications, vol. 201, p. 117148, Sep. 2022, doi: 10.1016/j.eswa.2022.117148.

[42]	J. Bamwenda, M. S. Özerdem, O. Ayyıldız, and V. Akpolat, “A Hybrid Deep Learning Framework for Accurate Cell Segmentation in Whole Slide Images Using YOLOv11, StarDist, and SAM2,” Bioengineering, vol. 12, no. 6, p. 674, Jun. 2025, doi: 10.3390/bioengineering12060674.

[43]	N. Kumar et al., “A Multi-Organ Nucleus Segmentation Challenge,” IEEE Transactions on Medical Imaging, vol. 39, no. 5, pp. 1380–1391, May 2020, doi: 10.1109/TMI.2019.2947628.

[44]	Z. Guo et al., “Reduce False-Positive Rate by Active Learning for Automatic Polyp Detection in Colonoscopy Videos,” in 2020 IEEE 17th International Symposium on Biomedical Imaging (ISBI), Apr. 2020, pp. 1655–1658. doi: 10.1109/ISBI45749.2020.9098500.

[45]	C. Cao, R. Wang, Y. Yu, H. Zhang, Y. Yu, and C. Sun, “Gastric polyp detection in gastroscopic images using deep neural network,” PLOS ONE, vol. 16, no. 4, p. e0250632, Apr. 2021, doi: 10.1371/journal.pone.0250632.

[46]	I. Pacal and D. Karaboga, “A robust real-time deep learning based automatic polyp detection system,” Computers in Biology and Medicine, vol. 134, p. 104519, Jul. 2021, doi: 10.1016/j.compbiomed.2021.104519.

[47]	J. Lee, J. Chae, and H. Cho, “Improvement of Colon Polyp Detection Performance by Modifying the Multi-scale Network Structure and Data Augmentation,” J. Electr. Eng. Technol., vol. 17, no. 5, pp. 3057–3065, Sep. 2022, doi: 10.1007/s42835-022-01191-3.

[48]	J. Wan, B. Chen, and Y. Yu, “Polyp Detection from Colorectum Images by Using Attentive YOLOv5,” Diagnostics, vol. 11, no. 12, Dec. 2021, doi: 10.3390/diagnostics11122264.

[49]	I. Pacal et al., “An efficient real-time colonic polyp detection with YOLO algorithms trained by using negative samples and large datasets,” Computers in Biology and Medicine, vol. 141, p. 105031, Feb. 2022, doi: 10.1016/j.compbiomed.2021.105031.

[50]	S. Durak et al., “Deep neural network approaches for detecting gastric polyps in endoscopic images,” Med Biol Eng Comput, vol. 59, no. 7–8, pp. 1563–1574, Aug. 2021, doi: 10.1007/s11517-021-02398-8.

[51]	Z. Qian, W. Jing, Y. Lv, and W. Zhang, “Automatic Polyp Detection by Combining Conditional Generative Adversarial Network and Modified You-Only-Look-Once,” IEEE Sensors Journal, vol. 22, no. 11, pp. 10841–10849, Jun. 2022, doi: 10.1109/JSEN.2022.3170034.

[52]	P. Carrinho and G. Falcao, “Highly accurate and fast YOLOv4-based polyp detection,” Expert Systems with Applications, vol. 232, p. 120834, Dec. 2023, doi: 10.1016/j.eswa.2023.120834.

[53]	A. Karaman et al., “Hyper-parameter optimization of deep learning architectures using artificial bee colony (ABC) algorithm for high performance real-time automatic colorectal cancer (CRC) polyp detection,” Applied Intelligence, vol. 53, no. 12, pp. 15603–15620, Nov. 2022, doi: 10.1007/s10489-022-04299-1.

[54]	A. Karaman et al., “Robust real-time polyp detection system design based on YOLO algorithms by optimizing activation functions and hyper-parameters with artificial bee colony (ABC),” Expert Systems with Applications, vol. 221, p. 119741, Jul. 2023, doi: 10.1016/j.eswa.2023.119741.

[55]	Y. Yu, K. Lin, J. Hong, R.-G. Tsai, and Y. Huang, “PD-YOLO: Colon Polyp Detection Model Based on Enhanced Small-Target Feature Extraction,” CMC, vol. 82, no. 1, pp. 913–928, 2025, doi: 10.32604/cmc.2024.058467.

[56]	S. Behzadi, D. Sharifrazi, B. Mesbahzadeh, J. H. Joloudari, and R. Alizadehsani, “A Lightweight and Robust Framework for Real-Time Colorectal Polyp Detection Using LOF-Based Preprocessing and YOLO-v11n,” Oct. 11, 2025, arXiv: arXiv:2507.10864. doi: 10.48550/arXiv.2507.10864.

[57]	N. B. Sureban, A. C. Ajish, Y. Abhinav, M. R. Prusty, and S. R. Patra, “A channel-efficient colorectal polyp detection using wavelet-enhanced reverse attention and custom neck YOLO architecture,” Biomedical Signal Processing and Control, vol. 119, p. 109909, Jun. 2026, doi: 10.1016/j.bspc.2026.109909.

[58]	S. Wang, J. Xie, Y. Cui, and Z. Chen, “Colorectal Polyp Detection Model by Using Super-Resolution Reconstruction and YOLO,” Electronics, vol. 13, no. 12, Jun. 2024, doi: 10.3390/electronics13122298.

[59]	A. Mert, A. Hakan Büyüklü, B. Bayram, and S. Erpolat Taşabat, “Multi-Dataset Training Strategy for Robust Polyp Detection With Clinical Validation Insights,” IEEE Access, vol. 13, pp. 162000–162008, 2025, doi: 10.1109/ACCESS.2025.3608668.

[60]	J. Li et al., “A colonic polyps detection algorithm based on an improved YOLOv5s,” Sci Rep, vol. 15, no. 1, p. 6852, Feb. 2025, doi: 10.1038/s41598-025-91467-1.

[61]	F. Xu et al., “Predicting Axillary Lymph Node Metastasis in Early Breast Cancer Using Deep Learning on Primary Tumor Biopsy Slides,” Front. Oncol., vol. 11, Oct. 2021, doi: 10.3389/fonc.2021.759007.

[62]	J. Bernal, F. J. Sánchez, G. Fernández-Esparrach, D. Gil, C. Rodríguez, and F. Vilariño, “WM-DOVA maps for accurate polyp highlighting in colonoscopy: Validation vs. saliency maps from physicians,” Computerized Medical Imaging and Graphics, vol. 43, pp. 99–111, Jul. 2015, doi: 10.1016/j.compmedimag.2015.02.007.

[63]	J. Silva, A. Histace, O. Romain, X. Dray, and B. Granado, “Toward embedded detection of polyps in WCE images for early diagnosis of colorectal cancer,” Int J CARS, vol. 9, no. 2, pp. 283–293, Mar. 2014, doi: 10.1007/s11548-013-0926-3.

[64]	R. Sapkota et al., “YOLOv12 to Its Genesis: A Decadal and Comprehensive Review of The You Only Look Once (YOLO) Series,” Feb. 21, 2025. doi: 10.1007/s10462-025-11253-3.

[65]	R. Khanam and M. Hussain, “A Review of YOLOv12: Attention-Based Enhancements vs. Previous Versions,” Apr. 16, 2025, arXiv: arXiv:2504.11995. doi: 10.48550/arXiv.2504.11995.

[66]	J. Redmon and A. Farhadi, “YOLO9000: Better, Faster, Stronger,” in 2017 IEEE Conference on Computer Vision and Pattern Recognition (CVPR), Jul. 2017, pp. 6517–6525. doi: 10.1109/CVPR.2017.690.

[67]	J. Redmon and A. Farhadi, “YOLOv3: An Incremental Improvement,” Apr. 08, 2018, arXiv: arXiv:1804.02767. doi: 10.48550/arXiv.1804.02767.

[68]	A. Bochkovskiy, C.-Y. Wang, and H.-Y. M. Liao, “YOLOv4: Optimal Speed and Accuracy of Object Detection,” Apr. 23, 2020, arXiv: arXiv:2004.10934. doi: 10.48550/arXiv.2004.10934.

[69]	G. Jocher and J. Qiu, Ultralytics YOLO. (2024). [Online]. Available: https://github.com/ultralytics/ultralytics

[70]	C. Li et al., “YOLOv6: A Single-Stage Object Detection Framework for Industrial Applications,” Sep. 07, 2022, arXiv: arXiv:2209.02976. doi: 10.48550/arXiv.2209.02976.

[71]	C.-Y. Wang, A. Bochkovskiy, and H.-Y. M. Liao, “YOLOv7: Trainable Bag-of-Freebies Sets New State-of-the-Art for Real-Time Object Detectors,” in 2023 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), Jun. 2023, pp. 7464–7475. doi: 10.1109/CVPR52729.2023.00721.

[72]	C.-Y. Wang, I.-H. Yeh, and H.-Y. M. Liao, “YOLOv9: Learning What You Want to Learn Using Programmable Gradient Information,” Feb. 29, 2024, arXiv: arXiv:2402.13616. doi: 10.48550/arXiv.2402.13616.

[73]	A. Wang et al., “YOLOv10: Real-Time End-to-End Object Detection,” Oct. 30, 2024, arXiv: arXiv:2405.14458. doi: 10.48550/arXiv.2405.14458.

[74]	Y. Tian, Q. Ye, and D. Doermann, “YOLOv12: Attention-Centric Real-Time Object Detectors,” Feb. 18, 2025, arXiv: arXiv:2502.12524. doi: 10.48550/arXiv.2502.12524.

[75]	M. Chileshe, M. Nyirenda, and J. Kaoma, “Early Detection of Sexually Transmitted Infections Using YOLO 12: A Deep Learning Approach,” Open Journal of Applied Sciences, vol. 15, no. 4, Art. no. 4, Mar. 2025, doi: 10.4236/ojapps.2025.154078.

[76]	Y.-H. Chen, “Strong Baseline: Multi-UAV Tracking via YOLOv12 with BoT-SORT-ReID,” Apr. 07, 2025, arXiv: arXiv:2503.17237. doi: 10.48550/arXiv.2503.17237.

[77]	X. Bu, Y. Wu, H. Lv, and Y. Yu, “Fast and Accurate Detection of Forty Types of Fruits and Vegetables: Dataset and Method,” Agriculture, vol. 15, no. 7, Art. no. 7, Jan. 2025, doi: 10.3390/agriculture15070760.

[78]	R. Sapkota, R. H. Cheppally, A. Sharda, and M. Karkee, “RF-DETR Object Detection vs YOLOv12 : A Study of Transformer-based and CNN-based Architectures for Single-Class and Multi-Class Greenfruit Detection in Complex Orchard Environments Under Label Ambiguity,” Apr. 17, 2025, arXiv: arXiv:2504.13099. doi: 10.48550/arXiv.2504.13099.

[79]	J. Ye, Y. Zhang, P. Li, Z. Guo, S. Zeng, and T. Wei, “Real-time dense small object detection model for floating litter detection and removal on water surfaces,” Marine Pollution Bulletin, vol. 218, p. 118189, Sep. 2025, doi: 10.1016/j.marpolbul.2025.118189.

[80]	J. Ma, Y. Zhou, Z. Zhou, Y. Zhang, and L. He, “Toward smart ocean monitoring: Real-time detection of marine litter using YOLOv12 in support of pollution mitigation,” Marine Pollution Bulletin, vol. 217, p. 118136, Aug. 2025, doi: 10.1016/j.marpolbul.2025.118136.

[81]	T. Cheng, L. Song, Y. Ge, W. Liu, X. Wang, and Y. Shan, “YOLO-World: Real-Time Open-Vocabulary Object Detection,” Feb. 22, 2024, arXiv: arXiv:2401.17270. doi: 10.48550/arXiv.2401.17270.

[82]	A. Wang, L. Liu, H. Chen, Z. Lin, J. Han, and G. Ding, “YOLOE: Real-Time Seeing Anything,” Mar. 10, 2025, arXiv: arXiv:2503.07465. doi: 10.48550/arXiv.2503.07465.

[83]	R. L. Portase, R. Tolas, and R. Potolea, “From Sensors to Insights: An Original Method for Consumer Behavior Identification in Appliance Usage,” Electronics, vol. 13, no. 7, Art. no. 7, Jan. 2024, doi: 10.3390/electronics13071364.

[84]	Y. Zhao et al., “DETRs Beat YOLOs on Real-time Object Detection,” in 2024 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), Jun. 2024, pp. 16965–16974. doi: 10.1109/CVPR52733.2024.01605.

[85]	W. Lv, Y. Zhao, Q. Chang, K. Huang, G. Wang, and Y. Liu, “RT-DETRv2: Improved Baseline with Bag-of-Freebies for Real-Time Detection Transformer,” arXiv.org. Accessed: Jun. 04, 2025. [Online]. Available: https://arxiv.org/abs/2407.17140v1

[86]	K. Blagec, G. Dorffner, M. Moradi, and M. Samwald, “A critical analysis of metrics used for measuring progress in artificial intelligence,” Nov. 08, 2021, arXiv: arXiv:2008.02577. doi: 10.48550/arXiv.2008.02577.

[87]	E.-R. Ardelean, R. L. Portase, R. Potolea, and M. Dînșoreanu, “A path-based distance computation for non-convexity with applications in clustering,” Knowl Inf Syst, Nov. 2024, doi: 10.1007/s10115-024-02275-4.